- Turn the ignition key ON THREE TIMES in succession: 
ON : OFF : ON : OFF : ON 
(within FIVE seconds)

- Count the number of times the check engine lamp on the dash flashes. The number of flashes represents the error code. There will be a short pause between the flashes representing the different digits of the code. Longer pauses separate individual trouble codes.

- Code 55 will be the last code to be displayed.

- These codes will also be displayed on the odometer, where they are much easier to read!

The Codes - 1997 Models: 


11 No crank reference signal detected during engine cranking. Intermittent loss of either camshaft or crankshaft position sensor. CKP sensor target windows have too much variation. 

12 Direct battery input to PCM was disconnected within the last 50 Key-on cycles. 

13 No difference recognized between the engine MAP reading and the barometric (atmosphere) pressure reading from start-up. 

14 MAP sensor input above or below acceptable voltage. 5 volt output to MAP sensor open. 

15 No vehicle speed sensor signal detected during road load conditions. 

17 Engine did not reach operating temperature within acceptable limits. Engine does not reach 20º F. within 5 minutes with a vehicle speed signal. 

21 Upstream oxygen sensor response slower than minimum required switching frequency. Upstream oxygen sensor heating element circuit malfunction. Downstream oxygen sensor heating element circuit malfunction. Downstream oxygen sensor input voltage maintained above the normal operating range. Oxygen sensor voltage too low, tested after cold start. (Upstream or Downstream) Left oxygen sensor input voltage maintained above the normal operating temperature. 

22 Engine coolant temperature sensor above or below acceptable voltage. 

23 Intake air temperature sensor input above or below acceptable voltage. 

24 Throttle position sensor input above or below acceptable voltage. TPS signal does not correlate to MAP sensor. 

25 A shorted or open condition detected in one or more of the idle air control motor circuits. Actual idle speed does not equal target idle speed. 

27 Injector #3, and/or 4, and/or 5, and/or 6 output driver does not respond properly to the control signal.

30 Sensor heater relay problem 

31 An open or shorted condition detected in the A/C clutch relay circuit. Insufficient or excessive vapor flow detected during evaporative emission system operation. 

33 An open or shorted condition detected in the duty cycle purge solenoid circuit. 

34 An open or shorted condition detected in the Speed Control vacuum or vent solenoid circuits. Speed control switch input below the minimum acceptable voltage.

36 Sensor heater relay problem

37 Relationship between engine speed and vehicle speed indicates no torque converter clutch engagement. An open or shorted condition detected in the torque converter part throttle unlock solenoid control circuit. Incorrect input state detected for the Park/Neutral switch. Auto only. 

41 An open or shorted condition in the generator field control circuit. 

42 An open or shorted condition detected in the auto shutdown relay circuit. An open condition detected in the ASD relay output circuit. An open or shorted condition detected in the fuel pump relay control circuit. An open circuit between PCM and fuel gauge sending unit. Circuit shorted to voltage between PCM and fuel gauge sending unit. No movement of fuel level sender detected. 

43 Peak primary circuit current not achieved with maximum dwell time. Misfire detected in one or more cylinders 1 thru 6. (4 and 6 cyls.) 

44 Battery temperature sensor in voltage above or below acceptable range. 

46 Battery voltage sense input above target charging voltage during engine operation. 

47 Battery voltage sense input below target charging during engine operation. Also, no significant change detected in battery voltage during active test of generator output circuit. 

51 A lean air/fuel mixture has been indicated by an abnormally rich correction factor. 

52 A rich air/fuel mixture has been indicated by an abnormally rich correction factor. 

53 PCM Internal fault condition detected. 

54 No camshaft signal detected during engine cranking. 

55 Completion of fault code display on Check Engine lamp. 

63 Unsuccessful attempt to write to an EEPROM location by the PCM. 

64 Catalyst efficiency below required level. (Same as code 72) 

65 Power steering high pressure seen at high speed. (2.5L only) 

72 Catalyst efficiency below required level. (Same as code 64) 

77 Malfunction detected with poser feed to speed control servo solenoids

106 MAP sensor voltage out of range detected at startup 

107 MAP sensor voltage too low 

108 MAP sensor voltage too high 

112 Intake air temperature sensor voltage low 

113 Intake air (charge) temperature sensor voltage high 

116 Coolant temparature sensor reading doesn't make 
sense 

117 Engine coolant temperature sensor voltage low 

118 Engine coolant temperature sensor voltage high 

121 Throttle position sensor and MAP sensor disagree with each other 

122 Throttle position sensor voltage low 

123 Throttle position sensor voltage high 

125 Taking too long to reach proper operating temparature and switch to energy-efficient mode 

130 Sensor heater relay problem 

131 Oxygen sensor seems to be shorted out or broken 

147 Oxygen sensor heater element not working properly (this device helps to reduce emissions more quickly) 

151 Oxygen sensor voltage problem - short circuit to ground? 

152 Oxygen sensor voltage problem - short circuit to active 12V? 

153 Oxygen sensor response too slow 

154 Oxygen sensor does not show either a rich or lean condition - may need replacement 

155 Oxygen sensor heater element not working properly 
(this device helps to reduce emissions more quickly) 

157 Oxygen sensor voltage problem - short circuit to ground? 

158 Oxygen sensor voltage problem - short circuit to active 12V? 

159 Oxygen sensor response too slow 

160 Oxygen sensor does not show either a rich or lean condition - may need replacement 

161 Oxygen sensor heater element not working properly (this device helps to reduce emissions more quickly) 

171 The oxygen sensor is saying that the system air/fuel mix is far too lean (too much fuel is being added as a correction). 

172 The oxygen sensor is saying that the system air/fuel mix is far too rich (too much air is being added as a correction). 

174 The oxygen sensor is saying that the system air/fuel mix is far too lean (too much fuel is being added as a correction). 

175 The oxygen sensor is saying that the system air/fuel mix is far too rich (too much air is being added as a correction). 

176 Flex fuel sensor can't be seen 

178 Flex fuel sensor problem 

179 Flex fuel sensor problem 

182 Compressed natural gas temperature sensor problem 

183 Compressed natural gas temperature sensor problem 

201 Injector #1 control circuit problem (open or shorted) 

202 Injector #2 control circuit problem (open or shorted) 

203 Injector #3 control circuit problem (open or shorted) 

204 Injector #4 control circuit problem (open or shorted) 

205 Injector #5 control circuit problem (open or shorted) 

206 Injector #6 control circuit problem (open or shorted) 

207 Injector #7 control circuit problem (open or shorted) 

208 Injector #8 control circuit problem (open or shorted) 

209 Injector #9 control circuit problem (open or shorted) 

210 Injector #10 control circuit problem (open or shorted) 

300 Misfire detected in multiple cylinders. 

301 Misfire detected in cylinder #1. 

302 Misfire detected in cylinder #2. 

303 Misfire detected in cylinder #3. 

304 Misfire detected in cylinder #4. 

305 Misfire detected in cylinder #5. 

306 Misfire detected in cylinder #6. 

307 Misfire detected in cylinder #7 

308 Misfire detected in cylinder #8. 

309 Misfire detected in cylinder #9. 

310 Misfire detected in cylinder #10. 

320 Crankshaft position sensor reference signal cannot be found while the engine is cranking. 

325 Knock sensor (#1) signal is wrong. 

330 Knock sensor (#2) signal is wrong. 

340 No camshaft signal being received by the computer 

350 A coil is drawing too much current. 

351 Coil #1 is not reaching peak current at the right time 

352 Coil #2 is not reaching peak current at the right time 

353 Coil #3 is not reaching peak current at the right time 

354 Coil #4 is not reaching peak current at the right time 

355 Coil #5 is not reaching peak current at the right time 

356 Coil #6 is not reaching peak current at the right time 

357 Coil #7 is not reaching peak current at the right time 

358 Coil #8 is not reaching peak current at the right time 

401 A required change in air/fuel mixture was not detected during diagnostic test. 

403 An problem was detected in the EGR solenoid control circuit. 

404 The EGR sensor's reported position makes no sense 

405 EGR position sensor voltage wrong. 

406 EGR position sensor voltage wrong. 

412 The secondary air solenoid control circuit seems bad (this is used for the aspirator). 

420 The catalyst seems inefficient (#1). 

432 The catalyst seems inefficient (#2). 

441 Evaporative purge flow system not working properly 

442 A leak has been detected in the evaporative system! 

443 Evaporative purge flow system solenoid not working properly 

455 A large leak has been detected in the evaporative system! 

456 A small leak has been detected in the evaporative system! 

460 The fuel level sender is not reporting any change over a long distance. Something seems wrong. 

461 The fuel level sender is not reporting any change over a long time. Something seems wrong. 

462 Fuel level sender voltage wrong. 

463 Fuel level sender voltage wrong. 

500 Haven't heard from the speed sensor lately. 

505 The idle speed air control motor doesn't seem to be working correctly. 

522 Oil pressure sensor problems 

523 Oil pressure sensor problems 

551 The power steering switch may not be working. (Neons: high pressure is showing up at high speed) 

600 Oh-oh! The coprocessors aren't talking to each other within the computer! 

601 Internal computer error! 

604 Internal computer error! (RAM check) 

605 Internal computer error! (ROM) 

615 Starter relay circuit problem 

622 Generator field control problem 

645 A/C clutch relay circuit problem. 

700 The automatic transmission computer or Aisin computer has a problem - ask it what's going on. I don't know. 

703 Brake switch circuit information seems wrong. 

711 Based on the transmission temperature and its operations, it looks like the transmission temperature sensor's gone bad. 

712 Transmission fluid temperature sensor voltage wrong. 

713 Transmission fluid temperature sensor voltage wrong. 

720 The Output Shaft Speed Sensor doesn't match the reported vehicle speed. 

740 The engine's running faster than it should for these speeds, so I think thetorque convertor clutch lock-up system is bad 

743 Torque converter clutch (part throttle unlock) solenoid circuit problem - shift solenoid C electrical fault 

748 Governor Pressure Solenoid circuit problem (Transmission relay circuit problem in Jeep RE transmissions) 

751 Overdrive override switch has been pressed for over five minutes. Just thought you should know. 

753 Overdrive solenoid control circuit problem (transmission relay circuit in Jeep RE transmissions.) 

756 Shift solenoid B (2-3) fault 

783 The overdrive solenoid can't go from 3rd gear to the overdrive gear. 

801 Transmission reverse gear lockout solenoid circuit problem! 

833 Problem with the clutch-released switch circuit? 

1192 Inlet air temperature sensor voltage is wrong 

1193 Inlet air temperature sensor voltage is wrong 

1194 Oxygen sensor heater performance is faulty 

1195 Oxygen sensor is slow 

1196 The oxygen sensor switched too slowly (bank 2). 

1197 The oxygen sensor switched too slowly (bank 1). 

1198 Radiator coolant temperature sensor voltage is wrong. 

1199 Radiator coolant temperature sensor voltage is wrong. 

1281 The engine is staying cold too long - check your thermostat. 

1282 The fuel pump relay circuit seems to be having a problem. 

1288 The intake manifold short runner tuning valve circuit seems to be having a problem. 

1289 There's a problem in the manifold tuning valve solenoid control circuit. 

1290 Compressed natural gas system pressure is too high 

1291 The heated air intake sensor does not seem to be working. 

1292 Natural gas pressure sensor issue 

1293 Natural gas pressure sensor issue 

1294 Can't get to target engine speed, check for vacuum leaks and idle speed motor issues. 

1295 The throttle position sensor doesn't seem to be getting enough electricity. 

1296 The MAP sensor doesn't seem to be getting enough electricity. 

1297 The MAP sensor doesn't change its reading when the engine is running! 

1298 During wide-open throttle, the engine runs lean. 

1299 MAP Sensor and Throttle Position Sensor signals don't match, check for a vacuum leak.

1388 Auto shutdown relay circuit problems? 

1389 No Z1 or Z2 voltage seen by the computer when the auto shutdown relay is used. 

1390 Cam and crank signals don't match - did the timing belt skip a tooth? 

1391 Sometimes, I can't see the crank or cam sensor signal. 

1398 I can see the Crank Sensor's signal when I prepare for Misfire Diagnostics. Try replacing it. 

1399 Problem in the Wait to Start Lamp circuit - (diesels only?) 

1403 EGR position sensor not getting (enough) voltage. 

1476 Too little secondary air injection during aspirator test. 

1477 Too much secondary air injection during aspirator test. 

1478 Battery temperature sensor voltage wrong. 

1479 Transmission fan relay circuit problems? 

1480 PCV solenoid circuit problems? 

1481 Transmission RPM pulse generator signal for misfire detection seems wrong. 

1482 Catalyst temperature sensor circuit shorted low. 

1483 Catalyst temperature sensor circuit shorted high. 

1484 The catalyst seems to be overheating! 

1485 Air injection solenoid circuit problems. 

1486 Pinched or blocked hose in the evaporative hose system. 

1487 Control circuit of the #2 high-speed radiator fan control relay is having problems. 

1488 Auxiliary 5-volt sensor feed is too low. 

1489 High speed radiator fan control circuit problem. 

1490 Low speed radiator fan control circuit problem. 

1491 Radiator fan control circuit problem (may be solid state relays as well as other circuits). 

1492 Ambient or battery temperature sensor voltage wrong 

1493 Ambient or battery temperature sensor voltage wrong 

1494 Leak Detection Pump (LDP) pressure switch problem - electrical or the pump itself. 

1495 Leak Detection Pump (LDP) pressure switch problem - the solenoid circuit. 

1496 Sensor feed is below an acceptable limit. (under 4v for 4 seconds - should be 5v). 

1498 High speed radiator fan control circuit problem. (#3 control relay) 

1594 Voltage too high in charging system. 

1595 Speed control vacuum or vent solenoid control circuits shorted or lost. 

1596 Speed control switch always high 

1597 Speed control switch always low 

1598 A/C pressure sensor voltage high 

1599 A/C pressure sensor voltage low 

1681 No messages received from the cluster control module (dashboard computer). 

1682 Charging system doesn't seem to be working well. Check alternator, etc. 

1683 Speed control servo power control circuit problem. 

1684 [/b]The battery has been disconnected within the last 50 starts. 

1685 Invalid key received from the Smart Key Immobilizer Module. 

1686 No messages received from the Smart Key Immobilizer Module. 

1687 No messages received from the Mechanical Instrument Cluster module. 

1693 The companion engine control module has shown a fault. 

1694 No messages received from the powertrain control module-Aisin transmission. 

1695 No messages received from the body control module. 

1696 Unsuccessful attempt to write to an EEPROM location! 

1697 Unsuccessful attempt to update Service Reminder Indicator (SRI or EMR) mileage! 

1698 No messages received from the electronic transmission control module or the Aisin transmission controller. 

1719 Transmission 2-3 gear lockout solenoid control circuit problem. 

1740 Either the tcc solenoid or overdrive solenoid systems doesn't seem to be making much sense. 

1756 Transmission control pressure not equal to target. (Mid pressure problem) 

1757 Transmission control pressure not equal to target. (Zero pressure problem) 

1762 The Governor Pressure Sensor input was too high or too low for 3 consecutive park/neutral calibrations. 

1763 The Governor Pressure Sensor input is too high 

1764 The Governor Pressure Sensor input is too low. 

1765 Open or short in the Transmission Relay control circuit. 

1899 The Park/Neutral switch seems to be stuck!